Electro Signs Ltd

97 Vallentin Road

London E17 3JJ

Est: 1952

Tel:	020 8521 8066

Fax: 	020 8520 8127

Email	electrosi@aol.com

Info@electrosigns.co.uk

Website	www.coldcathodelight.com

www.electrosigns.co.uk

###### ELECTRO LIGHT

LOW VOLTAGE COLD CATHODE -  PRODUCT SPECIFICATION

- Running at 100mA – 1234 lumens per meter (Triphospher 3000)

- Low voltage (below 1,000v) eliminates special wiring and fireman’s switches

- Traditional copper wound control gear up to 40 metres remote installation

- Long Life tubing of 45,000 burning hours

- 15mm to 25mm dia tubing with a choice of over 40 colours available including triphosphor coatings

- Oven pumped lamps for cleaner burning and higher maintained lumen output

- Fully dimmable

Low Voltage Cold Cathode Twin Lamp Transformer Features

- Single or twin lamp format – simplifying multiple lamp installations

- 40º Ambient temperature rating – ensuring long reliable life under normal 
    		operating conditions

- Resin encapsulation – protects components from danger and the environment

- Steel case – compliant with Directive EN 61000-3-2

###### TWIN LAMP TRANSFORMER ELECTRICAL SPECIFICATION

Each Twin Lamp Transformer controls up to 4 metres of cold cathode tubing (25mmdia/ blue gas) manufactured to suite the voltage required

Input Voltage					220 – 240V

Secondary Voltage				950V

Operating Voltage				600V

Operating Current			  	100mA

Manufactured to 				1S0 9001

Uncorrected Input Current			0.90A

Watt Consumption/Metre			25W

Corrected Input Current				0.6A

Size – Height 122mm Length 330mm Width 108mm

Weight 9kg